Understanding Protocols

## TCP/IP PROTOCOL SUITE

Communications between computers on a network is done through protocol suits. The most widely used and most widely available protocol suite is TCP/IP protocol suite. A protocol suit consists of a layered architecture where each layer depicts some functionality which can be carried out by a protocol. Each layer usually has more than one protocol options to carry out the responsibility that the layer adheres to. TCP/IP is normally considered to be a 4 layer system. The 4 layers are as follows :

- Application layer (Application Layer, Session Layer, Presentation Layer)

- Transport layer

- Network layer

- Network Interface Layer (Data link layer, Physical Layer)

### 1. Application layer

This is the top layer of TCP/IP protocol suite. This layer includes applications or processes that use transport layer protocols to deliver the data to destination computers.

At each layer there are certain protocol options to carry out the task designated to that particular layer. So, application layer also has various protocols that applications use to communicate with the second layer, the transport layer. Some of the popular application layer protocols are :

- HTTP (Hypertext transfer protocol)

- FTP (File transfer protocol)

- SMTP (Simple mail transfer protocol)

- SNMP (Simple network management protocol) etc

### 2. Transport Layer

This layer provides backbone to data flow between two hosts. This layer receives data from the application layer above it. There are many protocols that work at this layer but the two most commonly used protocols at transport layer are TCP and UDP.

TCP is used where a reliable connection is required while UDP is used in case of unreliable connections.

TCP divides the data(coming from the application layer) into proper sized chunks and then passes these chunks onto the network. It acknowledges received packets, waits for the acknowledgments of the packets it sent and sets timeout to resend the packets if acknowledgements are not received in time. The term ‘reliable connection’ is used where it is not desired to loose any information that is being transferred over the network through this connection. So, the protocol used for this type of connection must provide the mechanism to achieve this desired characteristic. For example, while downloading a file, it is not desired to loose any information(bytes) as it may lead to corruption of downloaded content.

UDP provides a comparatively simpler but unreliable service by sending packets from one host to another. UDP does not take any extra measures to ensure that the data sent is received by the target host or not. The term ‘unreliable connection’ are used where loss of some information does not hamper the task being fulfilled through this connection. For example while streaming a video, loss of few bytes of information due to some reason is acceptable as this does not harm the user experience much.

Internet Addressing

Internet Ports

Multipurpose Internet Mail Extensions (MIME) is an Internet standard that extends the format of email to support:

- Text in character sets other than ASCII

- Non-text attachments: audio, video, images, application programs etc.

- Message bodies with multiple parts

- Header information in non-ASCII character sets

Although MIME was designed mainly for SMTP, the content types defined by MIME standards are also of importance in communication protocols outside email, such as HTTP for the World Wide Web. Servers insert the MIME header at the beginning of any Web transmission. Clients use this content type or media type header to select an appropriate viewer application for the type of data the header indicates. Some of these viewers are built into the Web client or browser (for example, almost all browsers come with GIF and JPEG image viewers as well as the ability to handle HTML files).